Model: HT-CPE303  
          OPENWRT 300Mbps Wireless N 4G LTE Router
[image: image1.png]



Features
Cutting-edge 4G network – share internet access with up to 30 Wi-Fi devices and enjoy download speeds of up to 150 Mbps and Upload speed to 50Mbps
Plug a SIM card and play – no configurations needed, compatibility of SIM cards in 100+ countries are assured by years of field tests
Two advanced LTE antennas and Two WIFI antenna – enjoy stable and efficient connections to every device thanks to the two external antennas
Wi-Fi router mode – plug an Ethernet cable into the LAN/WAN port for flexible access as a backup option if you can't get a 4G connection
Description

1) Support 4G LTE wireless network technology.

2) Support  LTE TDD, LTE FDD Band.

3) Support SIM slot 

4) 802.11n WiFi standard

5) 1* WAN port, 2* LAN ports

6) Support up to 30 users

7) 300Mbps WiFi 

 Spefication
 
	Router hardware

	Chipset
	MT7628NN

	Power input port
	DC 12V---1A

	WAN Port
	1 x RJ45(10BaseT/100BaseTX with Auto-MDIX IEEE 802.3/802.3u)

	LAN Port
	2 x RJ45(10BaseT/100BaseTX with Auto-MDIX IEEE 802.3/802.3u)

	WPS/Reset Buttons
	Buttons: WPS ,Long press: reset(Used to restore the factory configuration of the router.)

	Power button
	Router's power switch (not required)

	LED indication
	1,power supply
2.System status
3,WIFI Indication
4,4G:Blue light indicates 4G status
5,WAN Interfaces;
6,LAN1;
7,LAN2;

	Storage
	SDRAM: 64MB     
Nor Flash: 8MB

	Wireless parameters

	WIFI Frequency range
	FCC: 2412-2462MHz(Ch1-Ch11)
ETSI: 2412-2472MHz(Ch1-Ch13)

	Mobile network frequency
	FDD: B1 B3 B5 B8 
TDD: B34 B38 B39 B40 B41
WCDMA: B1 B8

	Transmission rate
	11n:270/243/216/162/108/81/54/27Mbps
135/121.5/108/81/54/40.5/27/13.5Mbps
130/117/104/78/52/39/26/13Mbps
65/58.5/52/39/26/19.5/13/6.5Mbps

	
	11g:54/48/36/24/18/12/9/6M(Adaptive)

	
	11b:11/5.5/2/1M(Adaptive)

	Number of working channels
	13

	Spread Spectrum Technology
	DSSS(Direct Sequence Spread Spectrum)

	Data modulation method
	BPSK, QPSK, CCK and OFDM (BPSK/QPSK/16-QAM/ 64-QAM)

	Sensitivity @PER (error packet rate)
	270M:-61dBm@10% PER;130M: -61dBm@10% PER;108M:-61dBm@10% PER;54M:-65dBm@10% PER;11M:-85dBm@8% PER;6M:-88dBm@10% PER;1M:-90dBm@8% PER

	Transmission distance
	Up to 50 meters in the room and up to 200 meters in the outdoors (depending on the environment)

	RF Power
	11n  : 15±dBm;
11g  : 17±1dBm;
11b  : 19±1dBm(Typical value)

	LTE related hardware

	SIM Card Slot
	Rear flip SIM card Slot

	Antennas
	External 4 antenna, corresponding to 4G main antenna, wifi dual antenna

	Router software features

	IP Address
	Static or dynamic

	PPP over Ethernet
	YES

	The maximum number of users
	32 (Simultaneous Internet access)

	Routing Protocol
	Static route, RIP1/RIP2

	Configuration & Management
	Based on WEB interface management (HTTP)

	DHCP
	Support Server and Client

	Firewall protection
	Built-in NAT firewall


